Lightweight Restaurant Ordering and Hosted Payment System Solution Based on NFC and QR Code

I. Project Positioning and Core Design Principles

This solution integrates passive NFC stickers and static QR code table stands with a pure Web interface (H5) and third-party hosted checkout, establishing a lightweight self-ordering closed loop featuring zero-app installation, tap-to-access, and security compliance.

- Zero App Dependency: Powered by native OS capabilities (iOS Background Tag Reading and Android NFC Tag Dispatch), customers tap or scan to open the ordering web page directly in their default browser.

- Complete Decoupling of Operations and Funds: The proprietary backend only compiles order records. Actual fund settlements are fully handled by official third-party payment providers (PayPay, Apple Pay, Stripe), ensuring the system never accesses or stores sensitive payment credentials or account balances.

II. Hardware Selection and Reference Information

1. Core Material Selection

- NFC Chip Model: NTAG213 selected.

  - Technical Standard: Compliant with NFC Forum Type 2 and ISO/IEC 14443 Type A protocols, allowing driverless recognition and browser launch on both iOS and Android.

  - Storage Budget: Nominal capacity of 144 bytes (approx. 132 bytes usable for NDEF). A standard ordering URL (e.g., https://menu.example.com/?store_id=101&table=5) occupies only 40–60 bytes after URI protocol prefix compression (https:// compressed to 1 byte), providing ample headroom for security validation hash parameters.

  - Alternative Model: NTAG215 (504 bytes) is reserved only when excessively long transient authentication tokens are required. FeliCa chips are excluded due to prohibitive costs in a non-native-app architecture.

- Substrate and Enclosure Structure:

  - Early-Stage Minimalist Transition: Directly attach the NFC sticker to the reverse side of the printed QR code paper, sealed between two sheets of clear plastic film or soft acrylic with double-sided tape to achieve an ultra-low-cost setup.

  - Subsequent Batch Scale-up: Transition to premium graphic paper and rigid acrylic stands as operational demands scale.

  - Environmental Protection: If dining tables are constructed of iron/stainless steel or stands feature metal bases, standard NFC signals will be disrupted by eddy currents. In such scenarios, anti-metal tags equipped with a ferrite absorber layer must be deployed. Standard white labels or wet inlays suffice for wooden or plastic surfaces.

2. Hardware Procurement References

- Early Testing Phase:
Sourcing NTAG213 tags on Amazon Japan provides relatively low pricing and rapid turnaround for building prototypes:

  - Amazon Japan - NTAG213 Tag Procurement Link 1

  - Amazon Japan - NTAG213 Tag Procurement Link 2

- Mid-to-Late Batch Scale-up Phase:
Explore online wholesale platforms to secure cost-effective components or finished goods:

  - 1688 - NTAG213 Supplier Catalog

  - 1688 - Acrylic Table Stand Factory Source

The listed acrylic supplier serves international markets including Africa, Europe, South America, Southeast Asia, North America, Northeast Asia, and the Middle East, confirming export capability. International shipping and customs duties must be evaluated against overall order volume.

III. NFC Encoding, Verification, and Write-Protection Standards

1. Production Workflow via Android Device (Using NFC Tools)

- Prerequisites: Install NFC Tools on an NFC-enabled Android phone; confirm NFC is enabled in system settings.

- Record Creation: Open App → Navigate to Write tab → Tap Add a record → Select URL / URI → Enter complete ordering URL (e.g., https://menu.example.com/?table=1) → Confirm and save.

- Data Encoding: Tap Write, align the blank NTAG213 tag with the phone's NFC antenna zone until vibration and the green confirmation prompt (Write complete) appear.

- Verification: Return to the home screen while unlocked; tap the tag to confirm the default browser opens immediately to the intended landing page.

2. Critical Production Step: Read-Only Locking (Lock Bits)

- Security Risk: Tags ship in a rewriteable state by default. Deploying unlocked tags leaves them vulnerable to malicious URL replacement or phishing redirection.

- Implementation: After validating URL functionality, navigate to Other within NFC Tools and execute Lock tag / Make read-only (or check Lock tag in official NXP TagWriter).

- Technical Mechanism: Sets OTP lock bits on page 0x02 (Static Lock) and page 0x28 (Dynamic Lock). This action is strictly irreversible; once locked, the chip remains permanent read-only hardware.

IV. System Architecture and Hosted Payment Flow

1. End-to-End Business Flow

- Tap to Trigger: Customer touches the NTAG213 tag, launching the web ordering page with store and table parameters embedded.

- Order Assembly: Customer selects items; frontend computes total and requests backend order generation (e.g., Table 5, Order ID, 2,500 JPY).

- Invoking Official Payment:

  - PayPay Selection: Backend initiates a payment order via PayPay APIs to retrieve a Deep Link. The frontend executes the redirection, instantly opening the installed PayPay App to an official payment confirmation screen.

  - Apple Pay / Credit Card Selection: Frontend mounts standard components (e.g., Stripe Checkout). On iOS, native Apple Pay sheet slides up from the bottom of the display.

- User-Side Authorization:

  - Customer completes authorization within PayPay or via Face ID / Touch ID / power button confirmation on Apple Pay.

  - Transaction authentication is fully isolated within official provider infrastructure; the self-built platform never intercepts credentials or balances.

- Asynchronous Reconciliation:

  - Upon successful payment, official servers dispatch a cryptographically signed Webhook event to the backend.

  - Backend updates the order to paid status and dispatches the ticket to the kitchen printer.

2. Architectural Advantages

- Zero Financial Compliance Burden: No storage or handling of cardholder data, ensuring maximum exemption under PCI-DSS standards.

- Elimination of Transaction Disputes: Official authorization channels prevent man-in-the-middle exploits and forged payment vouchers.

- Rapid Development Cycle: Standard developer SDKs (PayPay for Developers, Stripe API) eliminate the need to design custom payment cryptography.